AVP Path Extraction Document

Target AVP: Digest-Algorithm
Message: Multimedia-Auth-Answer (MAA)
Interface: Cx (3GPP TS 29.228 / TS 29.229)
Extraction Date: 2026-03-10 13:51:53

AVP Path:

[Level 0] 3GPP-SIP-Auth-Data-Item (Grouped AVP)

-> [Level 1] SIP-Digest-Authenticate (Grouped AVP)

-> [Level 2] Digest-Algorithm

Document Content:

1. Message Command Definition (TS 29.229) - Command specification and AVP list

2. Message Procedure/Behavior (TS 29.228) - Business process description

3. AVP Path Definitions (TS 29.229) - Definition of each AVP in the path

================================================================================

Message Command Definition (TS 29.229)

### 6.1.8	Multimedia-Auth-Answer (MAA) Command

The Multimedia-Auth-Answer (MAA) command, indicated by the Command-Code field set to 303 and the 'R' bit cleared in the Command Flags field, is sent by a server in response to the Multimedia-Auth-Request command. The Experimental-Result AVP may contain one of the values defined in clause 6.2.

Message Format

< Multimedia-Auth-Answer > ::=  < Diameter Header: 303, PXY, 16777216 >

< Session-Id >

[ DRMP ]

{ Vendor-Specific-Application-Id }

[ Result-Code ]

[ Experimental-Result ]

{ Auth-Session-State }

{ Origin-Host }

{ Origin-Realm }

[ User-Name ]

[ OC-Supported-Features ]

[ OC-OLR ]

*[ Load ]

*[ Supported-Features ]

[ Public-Identity ]

[ SIP-Number-Auth-Items ]

*[SIP-Auth-Data-Item ]

*[ AVP ]

[ Failed-AVP ]

*[ Proxy-Info ]

*[ Route-Record ]

Message Procedure/Behavior (TS 29.228)

## 6.3	Authentication procedures

This procedure is used between the S-CSCF and the HSS to exchange information to support the authentication between the end user and the home IMS network. The procedure is invoked by the S-CSCF, corresponds to the combination of the operations Cx-AV-Req and Cx-AV-Req-Resp (see  TS 33.203 [3]) and is used:

-	To retrieve authentication vectors from the HSS.

-	To resolve synchronization failures between the sequence numbers in the UE and the HSS for authentication schemes that support this capability (e.g. IMS-AKA).

-	To promote the result of the NASS-level authentication to the IMS level.

-	To retrieve the IP-address secure binding information for GPRS-IMS-Bundled Authentication (GIBA) from the HSS.

This procedure is mapped to the commands Multimedia-Auth-Request/Answer in the Diameter application specified in  TS 29.229 [5]. Tables 6.3.1 through 6.3.7 detail the involved information elements.

Table 6.3.1: Authentication Request

| Information element name | Mapping to Diameter AVP | Cat. | Description |
| --- | --- | --- | --- |
| Public User Identity (See 7.2) | Public-Identity | M | This information element contains the Distinct Public User Identity of the user |
| Private User Identity (See 7.3) | User-Name | M | This information element contains the Private User Identity |
| Number Authentication Items (See 7.10) | SIP-Number-Auth-Items | M | This information element indicates the number of authentication vectors requested. Certain authentication schemes do not support more than one set of authentication vectors (e.g. SIP Digest, GIBA). |
| Authentication Data (See 7.9) | SIP-Auth-Data-Item | M | See Table 6.3.2 for the contents of this information element. |
| S-CSCF Name (See 7.4) | Server-Name | M | This information element contains the name (SIP URL) of the S-CSCF. |
| Routing Information (See 7.13) | Destination-Host | C | If the S-CSCF knows the HSS name this AVP shall be present. This information is available if the MAR belongs to an already existing registration, e.g. in case of the re-registration, where the HSS name is stored in the S-CSCF. The HSS name is obtained from the Origin-Host AVP, which is received from the HSS, e.g. included in the MAA command. This information may not be available if the command is sent in case of the initial registration. In this case the Destination-Host AVP is not present and the command is routed to the next Diameter node, e.g. SLF, based on the Diameter routing table in the client. |

Table 6.3.2: Authentication Data content in Authentication Request

| Information element name | Mapping to Diameter AVP | Cat. | Description |
| --- | --- | --- | --- |
| Authentication Scheme (See 7.9.2) | SIP-Authentication-Scheme | M | This information element indicates the authentication scheme. See 3GPP TS 29.229 [5] for a list of values |
| Authentication Context (See 7.9.7) | SIP-Authentication-Context | C | This information element shall contain authentication-related information relevant for performing the authentication. It shall be absent for IMS-AKA authentication schemes. |
| Authorization Information (See 7.9.4) | SIP-Authorization | C | This information element shall only be present for a request due to an IMS-AKA synchronization failure. If present, only IMS-AKA authentication schemes are allowed. |

Table 6.3.3: Void

Table 6.3.4: Authentication Request Response

| Information element name | Mapping to Diameter AVP | Cat. | Description |
| --- | --- | --- | --- |
| User Identity (See 7.2) | Public-Identity | C | Public User Identity. It shall be present when the result is DIAMETER_SUCCESS. |
| Private User Identity (See 7.3) | User-Name | C | Private User Identity. It shall be present when the result is DIAMETER_SUCCESS. |
| Number Authentication Items (See 7.10) | SIP-Number-Auth-Items | C | This information element indicates the number of authentication vectors delivered in the Authentication Data information element. It shall be present when the result is DIAMETER_SUCCESS. For SIP Digest, NASS Bundled authentication and GIBA this AVP shall be set to a value of 1. |
| Authentication Data (See 7.9) | SIP-Auth-Data-Item | C | If the SIP-Number-Auth-Items AVP is equal to zero or it is not present, then this information element shall not be present. See Table 6.3.5 for the contents of this information element. |
| Result (See 7.6) | Result-Code / Experimental-Result | M | This information element indicates the result of the operation. It shall be mapped to Result-Code AVP for errors defined in the Diameter base protocol (see IETF RFC 6733 [31]). It shall be mapped to Experimental-Result AVP for Cx/Dx errors. This information element is mapped to a grouped AVP which contains the 3GPP Vendor ID in the Vendor-Id AVP, and the error code in the Experimental-Result-Code AVP. |

Table 6.3.5: Authentication Data information element content in Authentication Request Response

| Information element name | Mapping to Diameter AVP | Cat. | Description |
| --- | --- | --- | --- |
| Item Number (See 7.9.1) | SIP-Item-Number | C | This information element shall only be present for IMS-AKA authentication schemes. This information element shall be present when there are multiple occurrences of the Authentication Data information element in the Authentication Request Response, and the order in which they should be processed is significant. In this scenario, Authentication Data information elements with a low Item Number information element value should be processed before Authentication Data information elements with a high Item Number information element value. |
| Authentication Scheme (See 7.9.2) | SIP-Authentication-Scheme | M | This information element indicates the authentication scheme. |
| Authentication Information (See 7.9.3) | SIP-Authenticate | C | This information element shall only be present for IMS-AKA authentication schemes. |
| Authorization Information (See 7.9.4) | SIP-Authorization | C | This information element shall only be present for IMS-AKA authentication schemes. |
| Confidentiality Key (See 7.9.5) | Confidentiality-Key | C | This information element shall be present for IMS AKA authentication schemes. It shall contain the confidentiality key. |
| Integrity Key (See 7.9.6) | Integrity-Key | C | This information element shall only be present for IMS-AKA authentication schemes. This information element shall contain the integrity key. |
| Digest Authenticate (See 7.9.8) | SIP-Digest-Authenticate | C | This information element shall only be present for SIP Digest authentication scheme. See Table 6.3.7 for contents of this grouped AVP. |
| Line Identifier (See 7.9.9) | Line-Identifier | C | This information element shall only be present for NASS Bundled authentication scheme. This information element contains fixed broadband access line identifier associated to the user. This information element can be repeated. |
| Framed IP Address (See 7.9.10) | Framed-IP-Address | C | This information element shall only be present for GPRS-IMS-Bundled authentication scheme. If the IP Address of the User is an IPv4 address, this AVP shall be included. |
| Framed IPv6 Prefix (See 7.9.11) | Framed-IPv6-Prefix | C | This information element shall only be present for GPRS-IMS-Bundled authentication scheme. If the IP Address of the User is an IPv6 address, this AVP shall be included. |
| Framed Interface Id (See 7.9.12) | Framed-Interface-Id | C | This information element shall only be present for GPRS-IMS-Bundled authentication scheme. If and only if the IP Address of the User is an IPv6 address and the Framed-IPv6-Prefix AVP alone is not unique for the user this AVP shall be included. |

Table 6.3.6: Void

Table 6.3.7: Digest Authenticate information element content – Response for SIP Digest

| Information element name | Mapping to Diameter AVP | Cat. | Description |
| --- | --- | --- | --- |
| Digest Realm (See 7.9.8.1) | Digest-Realm | M | This information element corresponds to the Realm parameter as defined in IETF RFC 7616 [33]. |
| Digest Algorithm (See 7.9.8.3) | Digest-Algorithm | O | This information element contains the algorithm as defined in IETF RFC 7616 [33]. If this information element is present, it shall contain "MD5". If neither the Digest Algorithm information element nor the Alternate Digest Algorithm information element are present, then the value "MD5" is assumed. (NOTE 1) |
| Digest QoP (See 7.9.8.4) | Digest-QoP | M | This information element contains the QoP as defined in IETF RFC 7616 [33]. This information element shall be set to a value of "auth" by the HSS. |
| Digest HA1 (See 7.9.8.5) | Digest-HA1 | M | This information element contains the H(A1) for algorithm "MD5" as defined in IETF RFC 7616 [33]. (NOTE 1) |
| Alternate Digest Algorithm (See 7.9.8.6) | Alternate-Digest-Algorithm | O | This information element contains the algorithm as defined in IETF RFC 7616 [33]. (NOTE 2) |
| Alternate Digest HA1 (See 7.9.8.6) | Alternate-Digest-HA1 | O | This information element contains the H(A1), as defined in IETF RFC 7616 [33]. (NOTE 2) |
| NOTE 1: The MD5 algorithm is only supported for backward compatibility and may only be provided within the Digest Algorithm information element. NOTE 2: If the Alternate Digest HA1 information element is present, the Digest HA1 information element shall also be present and the Digest HA1 information element shall contain the MD5 hash. The algorithm of the Alternate Digest HA1 information element shall be determined by the Alternate Digest Algorithm information element. |  |  |  |

Table 6.3.8: Void

Table 6.3.9: Void

### 6.3.1	Detailed behaviour

The HSS shall, in the following order (in case of an error in any of the steps the HSS shall stop processing and return the corresponding error code, see  TS 29.229 [5]):

1.	Check that the Private User Identity and the Public User Identity exist in the HSS. If not Experimental-Result-Code shall be set to DIAMETER_ERROR_USER_UNKNOWN.

2.	Check that the Public User Identity matches a distinct Public User Identity in the HSS. If it doesn't, the Experimental-Result-Code shall be set to DIAMETER_ERROR_USER_UNKNOWN.

3.	Check whether the Private and Public User Identities in the request are associated in the HSS. If not Experimental-Result-Code shall be set to DIAMETER_ERROR_IDENTITIES_DONT_MATCH.

4.	Check the authentication scheme indicated in the request, and

-	if it is "Unknown", check the authentication scheme stored in HSS. If it is neither NASS-Bundled authentication nor SIP Digest authentication, Experimental-Result-Code shall be set to DIAMETER_ERROR_AUTH_SCHEME_NOT_SUPPORTED.

-	if not, check that the authentication scheme indicated in the request is supported. If not Experimental-Result-Code shall be set to DIAMETER_ERROR_AUTH_SCHEME_NOT_SUPPORTED.

This step is only applicable for IMS-AKA authentication. If the request indicates there is a synchronization failure, the HSS shall compare the S-CSCF name received in the request to the S-CSCF name stored in the HSS:

-	If they are identical the HSS shall process AUTS as described in  TS 33.203 [3] and return the requested authentication information. The Result-Code shall be set to DIAMETER_SUCCESS.

5.	Check the registration status of the Public User Identity received in the request:

-	If it is registered, the HSS shall compare the S-CSCF name received in the request to the S-CSCF name stored in the HSS:

-	If they are different, the HSS shall overwrite the S-CSCF name. If IMS restoration procedures are supported and the S-CSCF reassignment pending flag is set, the HSS shall reset the flag and keep the S-CSCF restoration information associated with the Public User Identity. The HSS shall download SIP-Auth-Data-Item stored up to a maximum specified in SIP-Number-Auth-Items received in the command Multimedia-Auth-Request. If authentication scheme is neither NASS-Bundled nor GIBA, the HSS shall set the Public User Identity's authentication pending flag which is specific to the Private User Identity received in the request. The Result-Code shall be set to DIAMETER_SUCCESS.

-	If they are identical, the HSS shall download SIP-Auth-Data-Item stored up to a maximum specified in SIP-Number-Auth-Items received in the command Multimedia-Auth-Request. The Result-Code shall be set to DIAMETER_SUCCESS.

-	If it is unregistered (i.e. registered as a consequence of an originating or terminating request or there is an S-CSCF keeping the user profile stored) or not registered, the HSS shall compare the S-CSCF name received in the request to the S-CSCF name stored in the HSS:

-	If they are different or if there is no S-CSCF name stored in the HSS for any Public User Identity of the IMS subscription, the HSS shall store the S-CSCF name. The HSS shall download SIP-Auth-Data-Item stored up to a maximum specified in SIP-Number-Auth-Items received in the command Multimedia-Auth-Request. If authentication scheme is neither NASS-Bundled nor GIBA, the HSS shall set the Public User Identity's authentication pending flag which is specific to the Private User Identity which was received in the request. The Result-Code shall be set to DIAMETER_SUCCESS.

-	If they are identical, the HSS shall download SIP-Auth-Data-Item stored up to a maximum specified in SIP-Number-Auth-Items received in the command Multimedia-Auth-Request. If authentication scheme is neither NASS-Bundled nor GIBA, the HSS shall set the Public User Identity's authentication pending flag which is specific to the Private User Identity that was received in the request. The Result-Code shall be set to DIAMETER_SUCCESS.

Exceptions to the cases specified here shall be treated by HSS as error situations, the Result-Code shall be set to DIAMETER_UNABLE_TO_COMPLY. No authentication information shall be returned.

### 6.3.1	Detailed behaviour

The HSS shall, in the following order (in case of an error in any of the steps the HSS shall stop processing and return the corresponding error code, see  TS 29.229 [5]):

1.	Check that the Private User Identity and the Public User Identity exist in the HSS. If not Experimental-Result-Code shall be set to DIAMETER_ERROR_USER_UNKNOWN.

2.	Check that the Public User Identity matches a distinct Public User Identity in the HSS. If it doesn't, the Experimental-Result-Code shall be set to DIAMETER_ERROR_USER_UNKNOWN.

3.	Check whether the Private and Public User Identities in the request are associated in the HSS. If not Experimental-Result-Code shall be set to DIAMETER_ERROR_IDENTITIES_DONT_MATCH.

4.	Check the authentication scheme indicated in the request, and

-	if it is "Unknown", check the authentication scheme stored in HSS. If it is neither NASS-Bundled authentication nor SIP Digest authentication, Experimental-Result-Code shall be set to DIAMETER_ERROR_AUTH_SCHEME_NOT_SUPPORTED.

-	if not, check that the authentication scheme indicated in the request is supported. If not Experimental-Result-Code shall be set to DIAMETER_ERROR_AUTH_SCHEME_NOT_SUPPORTED.

This step is only applicable for IMS-AKA authentication. If the request indicates there is a synchronization failure, the HSS shall compare the S-CSCF name received in the request to the S-CSCF name stored in the HSS:

-	If they are identical the HSS shall process AUTS as described in  TS 33.203 [3] and return the requested authentication information. The Result-Code shall be set to DIAMETER_SUCCESS.

5.	Check the registration status of the Public User Identity received in the request:

-	If it is registered, the HSS shall compare the S-CSCF name received in the request to the S-CSCF name stored in the HSS:

-	If they are different, the HSS shall overwrite the S-CSCF name. If IMS restoration procedures are supported and the S-CSCF reassignment pending flag is set, the HSS shall reset the flag and keep the S-CSCF restoration information associated with the Public User Identity. The HSS shall download SIP-Auth-Data-Item stored up to a maximum specified in SIP-Number-Auth-Items received in the command Multimedia-Auth-Request. If authentication scheme is neither NASS-Bundled nor GIBA, the HSS shall set the Public User Identity's authentication pending flag which is specific to the Private User Identity received in the request. The Result-Code shall be set to DIAMETER_SUCCESS.

-	If they are identical, the HSS shall download SIP-Auth-Data-Item stored up to a maximum specified in SIP-Number-Auth-Items received in the command Multimedia-Auth-Request. The Result-Code shall be set to DIAMETER_SUCCESS.

-	If it is unregistered (i.e. registered as a consequence of an originating or terminating request or there is an S-CSCF keeping the user profile stored) or not registered, the HSS shall compare the S-CSCF name received in the request to the S-CSCF name stored in the HSS:

-	If they are different or if there is no S-CSCF name stored in the HSS for any Public User Identity of the IMS subscription, the HSS shall store the S-CSCF name. The HSS shall download SIP-Auth-Data-Item stored up to a maximum specified in SIP-Number-Auth-Items received in the command Multimedia-Auth-Request. If authentication scheme is neither NASS-Bundled nor GIBA, the HSS shall set the Public User Identity's authentication pending flag which is specific to the Private User Identity which was received in the request. The Result-Code shall be set to DIAMETER_SUCCESS.

-	If they are identical, the HSS shall download SIP-Auth-Data-Item stored up to a maximum specified in SIP-Number-Auth-Items received in the command Multimedia-Auth-Request. If authentication scheme is neither NASS-Bundled nor GIBA, the HSS shall set the Public User Identity's authentication pending flag which is specific to the Private User Identity that was received in the request. The Result-Code shall be set to DIAMETER_SUCCESS.

Exceptions to the cases specified here shall be treated by HSS as error situations, the Result-Code shall be set to DIAMETER_UNABLE_TO_COMPLY. No authentication information shall be returned.

AVP Path Definitions (TS 29.229)

[0] 3GPP-SIP-Auth-Data-Item (Grouped)

### 6.3.13	SIP-Auth-Data-Item AVP

The SIP-Auth-Data-Item is of type Grouped, and contains the authentication and/or authorization information for the Diameter client.

AVP format

SIP-Auth-Data-Item :: = < AVP Header : 612 10415 >

[ SIP-Item-Number ]

[ SIP-Authentication-Scheme ]

[ SIP-Authenticate ]

[ SIP-Authorization ]

[ SIP-Authentication-Context ]

[ Confidentiality-Key ]

[ Integrity-Key ]

[ SIP-Digest-Authenticate ]

[ Framed-IP-Address ]

[ Framed-IPv6-Prefix ]

[ Framed-Interface-Id ]

* [ Line-Identifier ]

* [AVP]

--------------------------------------------------------------------------------

-> [1] SIP-Digest-Authenticate (Grouped)

### 6.3.36	SIP-Digest-Authenticate AVP

The SIP-Digest-Authenticate is of type Grouped and it contains a reconstruction of either the SIP WWW-Authenticate or Proxy-Authentication header fields specified in IETF RFC 7616 [29].

AVP format

SIP-Digest-Authenticate ::= < AVP Header: 635 10415>

{ Digest-Realm }

[ Digest-Algorithm ]

{ Digest-QoP }

{ Digest-HA1}

[ Alternate-Digest-Algorithm ]

[ Alternate-Digest-HA1 ]

*[ AVP ]

--------------------------------------------------------------------------------

-> [2] Digest-Algorithm

### 6.3.39	Digest-Algorithm AVP

The Digest-Algorithm AVP is defined in IETF RFC 4740 [15] and contains values as defined in IETF RFC 7616 [29].

NOTE:	The MD5 algorithm is only supported for backward compatibility.

--------------------------------------------------------------------------------