AVP Path Extraction Document

Target AVP: DSR-Flags
Message: Delete Subscriber Data Request (DSR)
Extraction Date: 2026-03-10 13:36:45

AVP Path:

[Level 0] DSR-Flags

Document Content:

1. Message Command Definition - Technical specification and AVP list

2. Message Procedure/Behavior - Business process and behavior description

3. AVP Path Definitions - Detailed definition of each AVP in the path (in hierarchical order)

================================================================================

Message Command Definition

### 7.2.11	Delete-Subscriber-Data-Request (DSR) Command

The Delete-Subscriber Data-Request (DSR) command, indicated by the Command-Code field set to 320 and the 'R' bit set in the Command Flags field, is sent from HSS or CSS to MME or SGSN.

Message Format when used over the S6a/S6d application:

< Delete-Subscriber-Data-Request > ::=	< Diameter Header: 320, REQ, PXY, 16777251 >

< Session-Id >

[ DRMP ]

[ Vendor-Specific-Application-Id ]

{ Auth-Session-State }

{ Origin-Host }

{ Origin-Realm }

{ Destination-Host }

{ Destination-Realm }

{ User-Name }

*[ Supported-Features ]

{ DSR-Flags }

[ SCEF-ID ]

*[ Context-Identifier ]

[ Trace-Reference ]

*[ TS-Code ]

*[ SS-Code ]

[ eDRX-Related-RAT ]

*[ External-Identifier ]

*[ AVP ]

*[ Proxy-Info ]

*[ Route-Record ]

The SCEF-ID shall be present when the flag "Delete monitoring events" in DSR-Flags AVP is set.

Message Format when used over the S7a/S7d application:

< Delete-Subscriber-Data-Request > ::=	< Diameter Header: 320, REQ, PXY, 16777308 >

< Session-Id >

[ DRMP ]

[ Vendor-Specific-Application-Id ]

{ Auth-Session-State }

{ Origin-Host }

{ Origin-Realm }

{ Destination-Host }

{ Destination-Realm }

{ User-Name }

*[ Supported-Features ]

{ DSR-Flags }

*[ AVP ]

*[ Proxy-Info ]

*[ Route-Record ]

Message Procedure/Behavior

#### 5.2.2.2	Delete Subscriber Data

### 5.2.2.2.1	General

This procedure shall be used between the MME and the HSS and between the SGSN and the HSS, to remove some data of the HSS user profile stored in the MME or SGSN. The procedure shall be invoked by the HSS and it corresponds to the functional level operation Delete Subscriber Data (see 3GPP TS 23.401 [2]).

It shall be used to remove:

-	a subset (wich may or may not be the complete set of APN configurations) of the APN Configuration Profile for the subscriber from the SGSN or combined MME/SGSN;

-	a proper subset (i.e. a subset that is not equal to the complete set of APN configurations and does not contain the default APN configuration) of the APN Configuration Profile for the subscriber from the MME;

-	the regional subscription;

-	the subscribed charging characteristics;

-	Session Transfer Number for SRVCC;

-	trace data;

-	ProSe subscription data;

-	Reset-IDs;

-	MSISDN;

-	UE Usage Type;

-	V2X subscription data.

-	External Identifier(s).

If the HSS knows that the UE has attached to the MME and SGSN parts of the same combined MME/SGSN via both E-UTRAN and UTRAN/GERAN(refer to clause 5.2.1.1.2, 5.2.1.1.3 for further details), the HSS should invoke this procedure for a single time to remove some or all data of the HSS user profile stored in the combined MME/SGSN, i.e. not invoke this procedure for each of the MME and the SGSN registered respectively.

If the Node-Type-Indicator information has been previously received as cleared in the ULR-Flags and if the MME has not been registered for SMS during update location procedure for the MME, the HSS may skip any removal of the SMS related subscription data and consequently does not have to make use of the Delete Subscriber Data procedure to update the SMS subscription data in the MME.

This procedure is mapped to the commands Delete-Subscriber-Data-Request/Answer (DSR/DSA) in the Diameter application specified in clause 7.

Table 5.2.2.2.1/1 specifies the involved information elements for the request.

Table 5.2.2.2.1/2 specifies the involved information elements for the answer.

Table 5.2.2.2.1/1: Delete Subscriber Data Request

| Information element name | Mapping to Diameter AVP | Cat. | Description |
| --- | --- | --- | --- |
| IMSI | User-Name (See IETF RFC 6733 [61]) | M | This information element shall contain the user IMSI, formatted according to 3GPP TS 23.003 [3], clause 2.2. |
| Supported Features (See 3GPP TS 29.229 [9]) | Supported-Features | O | If present, this information element shall contain the list of features supported by the origin host. |
| DSR Flags (See 7.3.25) | DSR-Flags | M | This Information Element shall contain a bit mask. See 7.3.25 for the meaning of the bits. |
| Trace Reference (See 7.3.64) | Trace- Reference | C | This parameter shall contain the same value as used for the activation of the Trace Session. This element shall be present only if the "Trace Data Withdrawal" bit is set in the DSR-Flags. |
| Context Identifier (See 7.3.27) | Context-Identifier | C | This parameter shall identify the PDN subscription context or GPRS-PDP context that shall be deleted. This element shall be present only if the "PDN subscription contexts Withdrawal" bit or the "PDP context withdrawal" bit is set in the DSR-Flags. In the "PDN subscription contexts Withdrawal" case, the Context-Identifier shall not be associated with the default APN configuration. For the compatibility with the MAP protocol as defined in the 3GPP TS 29.002 [24], this parameter shall not have a value of zero. |
| TS Code List (See 7.3.100) | TS-Code | C | This parameter shall contain the teleservice codes that are to be deleted from the subscription. This element shall be present only if the "SMS Withdrawal" bit is set in the DSR-Flags and the SMS related teleservice codes are to be deleted. |
| SS Code List (See 7.3.87) | SS-Code | C | This parameter shall contain the supplementary service codes that are to be deleted from the subscription. This element shall be present only if the "SMS Withdrawal" bit is set or the "LCS Withdrawal" bit is set in the DSR-Flags. |
| SCEF-Id (See 3GPP TS 29.336 [54]) | SCEF-ID | C | This parameter shall contain the identity of the SCEF to which monitoring events that are to be deleted are associated. This element shall be present if the "Delete monitoring events" bit is set in the DSR-Flags. |
| eDRX Related RAT List | eDRX-Related-RAT | C | This parameter shall contain the RAT types for which the eDRX Cycle Lengths is to be deleted from the subscription. This element shall be present only if the "eDRX-Cycle-Length-Withdrawal" bit is set in the DSR-Flags and the corresponding eDRX Cycle Lengths are to be deleted. If the "eDRX-Cycle-Length-Withdrawal" bit is set in DSR-Flags, but the eDRX-Related-RAT AVP is absent in this command, the MME/SGSN shall delete the stored eDRX cycle lengths for all RATs. |
| External Identifiers | External-Identifiers | O | If present, this parameter shall contain the External Identifier(s) to be deleted from the subscriber data; the MME/SGSN shall also delete those monitoring events that include an External-Identifier AVP in their event configuration matching any of the External-Identifiers in this IE. This IE shall be absent if the "External-Identifier-Withdrawal" bit is not set in the DSR-Flags. If this IE is absent, and "External-Identifier-Withdrawal" bit is set in DSR-Flags, the MME/SGSN shall delete the default External-Identifier in the Subscription-Data AVP and it shall also delete all monitoring events that include an External-Identifier AVP in their event configuration. |

Table 5.2.2.2.1/2: Delete Subscriber Data Answer

| Information element name | Mapping to Diameter AVP | Cat. | Description |
| --- | --- | --- | --- |
| Supported Features (See 3GPP TS 29.229 [9]) | Supported-Features | O | If present, this information element shall contain the list of features supported by the origin host. |
| Result (See 7.4) | Result-Code / Experimental-Result | M | This IE shall contain the result of the operation. The Result-Code AVP shall be used to indicate success / errors as defined in the Diameter base protocol (see IETF RFC 6733 [61]). The Experimental-Result AVP shall be used for S6a/S6d errors. This is a grouped AVP which shall contain the 3GPP Vendor ID in the Vendor-Id AVP, and the error code in the Experimental-Result-Code AVP. The following errors are applicable in this case: - User Unknown |
| DSA Flags (See 7.3.26) | DSA-Flags | C | This Information Element shall contain a bit mask. See 7.3.26 for the meaning of the bits. |

### 5.2.2.2.2	Detailed behaviour of the MME and the SGSN

When receiving a Delete Subscriber Data request, the MME or SGSN shall check whether the IMSI is known.

If it is not known, a result code of DIAMETER_ERROR_USER_UNKNOWN shall be returned.

If it is known, but the Context-Identifier is associated with the default APN configuration, the MME shall not delete the PDN subscription context, and return an error with a Result-Code set to DIAMETER_UNABLE_TO_COMPLY. Otherwise, the MME or SGSN shall delete the corresponding data according to the indication as sent in the request, and acknowledge the Delete Subscriber Data message by returning a Delete Subscriber Data Answer.

If an MME receives a Delete Subscriber Data Request with the "Complete APN Configuration Profile Withdrawal" bit set in the DSR-Flags AVP, it shall return an error with a Result-Code set to DIAMETER_UNABLE_TO_COMPLY.

If the deletion of the subscription data succeeds in the MME or SGSN, the Result-Code shall be set to DIAMETER_SUCCESS.

If the Regional Subscription is deleted from the subscription data, the SGSN shall check for its routing areas whether they are allowed or not. If the entire SGSN area is restricted, SGSN shall report it to the HSS by returning the "SGSN Area Restricted" indication within the DSA flags.

If the EPS Subscription Data is deleted from the subscription data, the MME or SGSN shall check whether all EPS Subscription Data for the subscriber is deleted or if only a subset of the stored EPS Subscription Data for the subscriber is deleted, the MME or SGSN may then deactivate the associated affected active EPS bearers.

If the Subscribed Charging Characteristics are deleted from the subscription data, the Gn/Gp-SGSN shall maintain the existing Subscribed Charging Characteristics throughout the lifetime of the existing MM and PDP contexts, see 3GPP TS 32.251 [33].

If the Subscribed Charging Characteristics are deleted from the subscription data, the MME or S4-SGSN shall maintain the existing Subscribed Charging Characteristics throughout the lifetime of the existing IP CAN bearer, see 3GPP TS 32.251 [33].

If the MSISDN is deleted from the subscription data, the MME or SGSN shall maintain the existing MSISDN throughout the lifetime of the existing PDN connections that were established prior to the deletion of the MSISDN (i.e., other network nodes, such as PDN-GW, are not informed of such deletion for the existing PDN connections). The MME/SGSN shall also delete those monitoring events related to the deleted MSISDN.

If the MME or SGSN cannot fulfil the received request for other reasons, e.g. due to a database error, it shall set the Result-Code to DIAMETER_UNABLE_TO_COMPLY. In this case, the MME or SGSN shall mark the subscription record "Subscriber to be restored in HSS".

If trace data are deleted from the subscription data, the MME or SGSN shall deactivate the Trace Session identified by the trace reference. For details, see 3GPP TS 32.422 [23].

If External Identifiers are requested to be deleted from the subscription data, the MME/SGSN shall check whether any of the identifiers to be deleted match the default External-Identifier provided by HSS in the Subscrition-Data AVP (unless all External Identifiers are requested to be deleted from the subscription); in such case, the MME/SGSN shall reject the request and return an error with a Result-Code set to DIAMETER_UNABLE_TO_COMPLY (if default External Identifier is wanted to be deleted, no External Identifier must be provided in the request). The MME/SGSN shall also delete those monitoring events related to the deleted External Identifiers, or all monitoring events associated to any External Identifier if default External Identifier is deleted.

### 5.2.2.2.3	Detailed behaviour of the HSS

The HSS shall make use of this procedure to remove deleted subscription data from the MME or SGSN.

The HSS shall make use of this procedure to remove deleted GPRS Subscription Data from the SGSN or combined MME/SGSN if the GPRS-Subscription-Data-Indicator information has been previously received as set in the ULR-Flags during update location procedure for the MME.

The HSS shall not set the "Complete APN Configuration Profile Withdrawal" bit in the DSR-Flags AVP when sending a Delete Subscriber Data Request to an MME, since the default APN shall always be present in an MME.

When receiving a Delete Subscriber Data Answer with "SGSN Area Restricted" the HSS shall set the SGSN area restricted flag as "SGSN area restricted".

AVP Path Definitions

[0] DSR-Flags

### 7.3.25	DSR-Flags

The DSR-Flags AVP is of type Unsigned32 and it shall contain a bit mask. The meaning of the bits is defined in table 7.3.25/1:

Table 7.3.25/1: DSR-Flags

| Bit | Name | Description |
| --- | --- | --- |
| 0 | Regional Subscription Withdrawal | This bit, when set, indicates that Regional Subscription shall be deleted from the subscriber data. |
| 1 | Complete APN Configuration Profile Withdrawal | This bit, when set, indicates that all EPS APN configuration data for the subscriber shall be deleted from the subscriber data. This flag only applies to the S6d interface. |
| 2 | Subscribed Charging Characteristics Withdrawal | This bit, when set, indicates that the Subscribed Charging Characteristics have been deleted from the subscription data. |
| 3 | PDN subscription contexts Withdrawal | This bit, when set, indicates that the PDN subscription contexts whose identifier is included in the Context-Identifier AVP shall be deleted. (Note 1) |
| 4 | STN-SR | This bit, when set, indicates that the Session Transfer Number for SRVCC shall be deleted from the subscriber data. |
| 5 | Complete PDP context list Withdrawal | This bit, when set, indicates that all PDP contexts for the subscriber shall be deleted from the subscriber data. |
| 6 | PDP contexts Withdrawal | This bit, when set, indicates that the PDP contexts whose identifier is included in the Context-Identifier AVP shall be deleted. (Note 2) |
| 7 | Roaming Restricted due to unsupported feature | This bit, when set, indicates that the roaming restriction shall be deleted from the subscriber data in the MME or SGSN. |
| 8 | Trace Data Withdrawal | This bit, when set, indicates that the Trace Data shall be deleted from the subscriber data. |
| 9 | CSG Deleted | This bit, when set, indicates that - the "CSG-Subscription-Data from HSS" shall be deleted in the MME or SGSN when received over the S6a or S6d interface - the "CSG-Subscription-Data from CSS" shall be deleted in the MME or SGSN when received over the S7a or S7d interface. |
| 10 | APN-OI-Replacement | This bit, when set, indicates that the UE level APN-OI-Replacement shall be deleted from the subscriber data. |
| 11 | GMLC List Withdrawal | This bit, when set, indicates that the subscriber's LCS GMLC List shall be deleted from the MME or SGSN. |
| 12 | LCS Withdrawal | This bit, when set, indicates that the LCS service whose code is included in the SS-Code AVP shall be deleted from the MME or SGSN. |
| 13 | SMS Withdrawal | This bit, when set, indicates that the SMS service whose code is included in the SS-Code AVP or TS-Code AVP shall be deleted from the MME or SGSN. |
| 14 | Subscribed periodic RAU-TAU Timer Withdrawal | This bit, when set, indicates that the subscribed periodic RAU TAU Timer value shall be deleted from the subscriber data. |
| 15 | Subscribed VSRVCC Withdrawal | This bit, when set, indicates that the Subscribed VSRVCC shall be deleted from the subscriber data. |
| 16 | A-MSISDN Withdrawal | This bit, when set, indicates that the additional MSISDN, if present, shall be deleted from the subscriber data. |
| 17 | ProSe Withdrawal | This bit, when set, indicates that the ProSe subscription data shall be deleted from the MME or combined MME/SGSN. |
| 18 | Reset-IDs | This bit, when set, indicates that the set of Reset-IDs shall be deleted from the MME or SGSN. |
| 19 | DL-Buffering-Suggested-Packet-Count Withdrawal | This bit, when set, indicates that the DL-Buffering-Suggested-Packet-Count shall be deleted in the MME or SGSN. |
| 20 | Subscribed IMSI-Group-Id Withdrawal | This bit, when set, indicates that all subscribed IMSI-Group-Id(s) shall be deleted in the MME or SGSN. |
| 21 | Delete monitoring events | This bit when set indicates to the MME or SGSN to delete all the Monitoring events for the subscriber which are associated with the provided SCEF-ID. |
| 22 | User Plane Integrity Protection Withdrawal | This bit, when set, indicates to the SGSN that User Plane Integrity Protection may no longer be required when GERAN is used. The MME shall ignore it. |
| 23 | MSISDN Withdrawal | This bit, when set, indicates that the MSISDN shall be deleted from the subscriber data. It is also used by the MME/SGSN to delete those monitoring events created using the MSISDN. |
| 24 | UE Usage Type Withdrawal | This bit, when set, indicates to the MME or SGSN that the UE Usage Type shall be deleted from the subscription data. |
| 25 | V2X Withdrawal | This bit, when set, indicates that the V2X subscription data shall be deleted from the MME or combined MME/SGSN. |
| 26 | External-Identifier-Withdrawal | This bit, when set, indicates that the External-Identifier shall be deleted from the subscriber data. It is also used by the MME/SGSN to delete those monitoring events created using the removed External Identifier or all monitoring events created for any External Identifier in case of removing the default External Identifier. |
| 27 | Aerial-UE-Subscription Withdrawal | This bit, when set, indicates that the Aerial UE subscription shall be deleted from the subscriber data. |
| 28 | Paging Time Window Subscription Withdrawal | This bit, when set, indicates that the Paging Time Window subscription shall be deleted from the subscriber data. |
| 29 | Active-Time-Withdrawal | This bit, when set, indicates that the Active Time used for PSM shall be deleted from the subscriber data. |
| 30 | eDRX-Cycle-Length -Withdrawal | This bit, when set, indicates that the eDRX-Cycle-Length shall be deleted from the subscriber data. . If the eDRX-Related-RAT is present in the DSR command, only the eDRX Cycle Length for indicated RAT types shall be deleted. Otherwise, the entire eDRX Cycle Length subscription for all RAT types shall be deleted. |
| 31 | Service-Gap-Time-Withdrawal | This bit, when set, indicates that the Service Gap Time shall be deleted from the subscriber data. |
| Note 1: If the Complete APN Configuration Profile Withdrawal bit is set, this bit should not be set. Note 2: If the Complete PDP context list Withdrawal bit is set, this bit should not be set. Note 3: Bits not defined in this table shall be cleared by the sending HSS and discarded by the receiving MME or SGSN. Note 4: Bits 3 and 6 are excluding alternatives and shall not both be set. Note 5: When this AVP is transferred over the S7a/S7d interface, only the bit 9 (CSG Deleted) is meaningful, other bits shall be cleared. |  |  |

--------------------------------------------------------------------------------